SECTION  - 1000

GENERAL REQUIREMENTS

1001	GENERAL

1002	TEMPORARY FACILITIES

1003	PAYMENT OF WORK

1001 	GENERAL

01. 	LOCATION

The Project is located at MNJ Road near village Bakriyal, District Mansehra.

02. 	SITE OF WORKS

The site of the works is the area for the Earthwork, Construction of Roads, Drainage, Water Supply & Sewerage System within the boundaries and limits shown on the drawings and such additional areas adjacent thereto as may be designated by the Engineer from time to time for the construction to be performed under the contract.

The Employer will give to the Contractor possession of as much of the area designated and defined as the site and is shown on the drawings in parts or in full as may be required to implement the works when the Engineer's Order to commence work is given.

03. 	CODES AND STANDARDS

A. 	Applicable Standards

Except as otherwise provided by these Specifications or the Drawings, all materials, equipment and fabrication and testing thereof shall conform to the latest applicable standards and codes referred in the Specifications by use of the abbreviations explained below:

ACI 		- American Concrete Institute (USA)

AISI 		- American Iron and Steel Institute (USA)

AISC		- American Institute of Steel Construction (USA)

ANSI		- American National Standard Institute (USA)

ASTM		- American Society for Testing and Materials (USA)

AASHTO	 -American Association of State Highway & Transportation Officials.

AWS 		- American Welding Society (USA)

BS 		- British Standards (UK)

CP 		- Codes of Practice (UK)

PS 		- Pakistan Standards (Pak)

SSPC 		- Steel Structures Painting Council (USA)

UBC 		- Uniform Building Code (USA)

USBR 		- United States Bureau of Reclamation (USA)

MUTCD	 - Manual on Uniform Traffic Control Devices (USA)

If the Contractor, at any time and for any reason, wishes to deviate from the above standards or desires to use material or equipment not covered by the above standards, he shall state the exact nature of the changes, the reason for making the change and shall submit complete specifications of the materials and equipment to the Engineer for approval.

B. 	Standards other than those Specified

Where requirements for materials or equipment are specified by reference to a standard which has its origin in one country, it is not the intention to restrict the requirements solely to that standard and that country. Other standards, including standards of other countries, will be accepted provided the requirements thereof, in the sole opinion of the Engineer, are at least equal to the requirements of the standard specified. The Contractor may propose to the Engineer an equivalent standard other than that specified, in which case he shall submit the proposed standard and all other information required and shall submit written proof that his proposed standard is equivalent in all significant respects to the standard specified. All submissions must be made in the English language.

C. 	Codes and Standards at Site

The Contractor shall supply and have at his site office :-

a)	Copies of all latest editions of codes and standards referred to in these Specifications or equivalent codes and standards as approved by the Engineer.

b) 	Catalogues and published recommendations from manufacturers supplying products and materials for the project.

c) 	The Contractor shall provide manufacturer's or supplier's materials which must meet the requirements of a specific code or standard as stated in these Specifications.

04. 	UNITS OF MEASUREMENTS

The FPS and MKS Systems of Units shall be used throughout the Project.

05. 	PLANT, EQUIPMENT AND TOOLS

The Contractor shall provide at his cost modern and in good condition plant, equipment and tools, adequate and befitting to the nature, magnitude and size of this Contract, in strict compliance with the requirements of the General Conditions of Contract and Technical Specifications.

06. 	STORAGE & HANDLING FACILITIES

The Contractor shall make his own arrangements for providing the necessary space for the storage of plant, equipment and materials and for Contractor's temporary office, in and around the site of works, during the currency of the Contract.

07. 	FIELD LABORATORY AND TESTING

A. 	General

The Contractor shall provide and maintain a field laboratory equipped with approved equipment to perform all the tests required by the Engineer. The quality control testing shall be performed by the Contractor's competent personnel in accordance with a site testing and quality control programme to be established by the Contractor and approved by the Engineer. The Engineer may however, require certain tests to be performed in any other laboratory designated by him.

The Contractor shall provide laboratory helpers to the Engineer for testing. The laboratory shall be run by a qualified material Engineer and Laboratory Technician to be employed by the Contractor.

The Field Laboratory, including all equipment and staff shall be placed at the disposal and direction of the Engineer during the Contract.

The Contractor shall keep a complete record of all quality tests performed on site.

All quality control and tests shall be carried out in accordance with applicable standards and codes.

B. 	Field Laboratory Equipment Requirements

The Laboratory shall be equipped with new unused and latest Equipment to perform tests as per Technical Specifications and General Conditions of Contract. Additional equipment/materials shall be supplied by the Contractor as and when required by the Engineer to perform any specified test, at no additional cost to the Employer.

The laboratory shall also be equipped with new unused furniture, fittings and fixtures. If any equipment, furniture, fitting or fixture becomes unserviceable for any reason what so ever, the Contractor shall promptly replace the same as and when directed by the Engineer.

# C. 	Method of Payment

The cost of providing running and maintenance of the laboratory, equipment, materials and staff, testing charges for materials supplied by the Employer and all other tests to be performed in any other laboratory designated by the Engineer shall be deemed to be included in the quoted unit rates of other items of the Bill of Quantities and no separate claim for payment on this account shall be entertained by the Engineer.

In case the Contractor does not provide the specified equipment and testing facility, cost of testing plus 100 percent overheads shall be recovered from his bills.

08. 	SURVEYING INSTRUMENTS

A. 	General

Contractor shall use state of the art surveying equipment. All surveying equipment shall be in good working condition. The quantity of all survey equipment shall be maintained throughout the Contract Period and replaced by the Contractor free of charge in case of damage or loss. The survey equipment shall be supplied to the Engineer at the same time when the Contractor takes over the site and provides the temporary office of the Engineer.

The Survey equipment shall be placed at the disposal and direction of the Engineer during the Contract.

The Contractor shall provide required number of qualified Surveyors and Survey Helpers to the Engineer's Representative for the surveying work.

B. 	Surveying Equipment Required

The Contractor shall provide and maintain the following surveying equipment for the Exclusive use by the Engineer's Representative at site.

| a) Electronic distance measuring device complete with (400 grades) Universal Theodolite with data logger, aiming head and standard equipment supplied by the manufacturer, including tripod, control unit stand, batteries, charging unit, pole reflector, single prism reflector, three, six and nine prism reflectors complete with tripods and Traverse Equipment for Theodolite. | 2 Nos. |
| --- | --- |

b. 	Automatic Levels with tripods 					 4  Nos.

c. 	Measuring tapes 50 m long 					 8  Nos.

d. 	Measuring tapes 20 m long 					 8  Nos.

e. 	Steel measuring tapes 5 m long 				             12 Nos.

f.	Measuring poles w/tripods 					18 Nos.

g.	Measuring steel staff (at least 4 meter long)			 4  Nos.

All miscellaneous tools, equipment and materials required in surveying.

The Contractor shall provide adequate supplies of the following e materials, i.e. pencil, rubbers and inks drawing papers, pegs, brushes and paints as required by the Engineer's Representative.

09. 	CONSTRUCTION DRAWINGS

After award of the Contract, the Tender Drawings will be replaced by the Drawings issued for Construction including Supplementary Specifications if necessary. Such Drawings and Specifications shall be deemed to be Further Drawings and Instructions referred in the Conditions of Contract. The Drawings Issued for Construction will include Tender Drawings reissued, Tender Drawings as may be modified, and additional drawings as required. The Drawings issued for Construction will be the drawings from which shop, reinforcing steel detail, erection, concrete placing, formwork, or other construction detail drawings shall be prepared by the Contractor. The work shall be executed in conformity with the Drawings Issued for Construction from time to time in accordance with the sequence and schedule of construction.

The Contractor shall check all Construction Drawings carefully as soon as practicable after receipt thereof, and shall promptly advise the Engineer of any errors or inconsistencies discovered.

10. 	SHOP DRAWINGS AND SCHEDULES

A. 	Shop/Reinforcement Drawings and Bar Bending Schedule

All shop drawings required for the work(except such drawings that are specified to be furnished by others under separate supply contracts) including reinforcing steel detailing, bending and cutting drawings/bar bending schedules, field erection and layout and construction detail drawings shall be furnished by the Contractor for approval of the Engineer.

Written drawing matter shall be in English. If more detailed drawings are necessary to complete any part of the work, such detailed drawings shall

All drawings shall be complete and shall be submitted in due time and in logical order to facilitate proper coordination. If required by the Engineer, the Contractor shall also submit detailed design calculations to substantiate the shop drawings.

Drawings showing proposed methods of construction and other drawings additional to those referred to herein above, required by the Specifications, shall also be submitted to the Engineer for approval.

B. 	Submittals

Except as otherwise specified, four (4) white copies of each drawing for approval or review shall be furnished as herein required. Within fifteen calendar days after receipt, the Engineer will send one copy to the Contractor marked Approved, Approved Except as Noted, or Returned for Noted which shall authorize the Contractor to proceed with the work covered by such drawings subject to the corrections, if any, indicated thereon. When prints of drawings have been Returned for Correction, the Contractor shall make the necessary revisions on the drawings and shall resubmit prints for revisions made during the life of the Contract shall be shown by number, date and subject in a revision block. The size of the shop drawings shall be same as the tender drawings.

All of the applicable requirements of this sub-section with reference to drawings to be prepared by the Contractor shall apply equally to catalogues, illustrations, printed specifications, or other data submitted for approval.

Where adjoining work requires shop drawings the Contractor shall prepare and submit composite shop drawings which shall show and define the work under all affected trades. If the Contractor installs work before coordinating with other trades so as to cause interference with work of those trades, he shall make changes necessary to correct the conditions without extra cost to the Employer.

No changes shall be made by the Contractor in the resubmitted shop drawings in excess of the corrections spelled out by the Engineer and in a separate note on the shop drawings.

No work in the shop shall be started and no material or equipment ordered until the Engineer has approved the shop drawings. It shall be the responsibility of the Contractor to submit the shop drawings on a schedule that allows reasonable time for checking and approval and subsequent fabrication. Failure to submit shop drawings in ample time for checking, correcting, and rechecking will not justify a delay in time for completion of work.

11. 	CONSTRUCTION SCHEDULE AND PROGRESS REPORTS

A Construction schedule shall be maintained in accordance with the provisions of the General Conditions of Contract.

The schedule shall be accompanied with sufficient data and information including all necessary particulars of construction plant, equipment, machinery, temporary Works, arrival of plant, equipment at site and their installation, method of operation, work forces employed, etc., for all activities of the Works.

Should the Engineer/Engineer’s Representative consider any alteration or addition in the Programme and time schedule, the Contractor shall conform thereto without any cost to the Employer.

Whenever necessary and wherever the progress of the actual works shows departure, the programme and time schedule shall be updated and submitted to the Engineer for his approval.

The Contractor shall submit weekly three copies & monthly six copies of progress reports as per provision of GC-II in a format as agreed by the Engineer. The reports should include updated bar charts, schedules and photographs (in original) taken at appropriate stages of construction of works, a construction schedule indicating the progress in percentage, a description of all works carried out since the last report and a description of the works planned for the next two months, sufficiently detailed to enable the Engineer/Engineer’s Representative to determine his programme of inspection and testing.

Progress meetings shall be held at site weekly or at intervals as directed by the Engineer. Representatives of the Contractor, who should be the director or partner, the Engineer, Engineer’s Representative or Resident Engineer and if required, the Employer shall review the progress achieved and discuss the plans and requirements for the future.

12. 	SAMPLES

The Contractor shall furnish for approval of the Engineer with reasonable promptness all samples as directed by the Engineer/Engineer’s Representative or specifically called for in these Specifications. The Engineer shall check and approve such samples with reasonable promptness only for conformance with the design concept of the Works and for compliance with the information given in the Contract Documents. All work shall be in accordance with approved samples.

Samples shall be furnished so as not to delay construction or fabrication, allowing the Engineer reasonable time for consideration of the sample submitted.

Each sample shall be properly labeled with the name and quality of the material, manufacturer's name, name of the project, the Contractor's name and the date of submission, and the Specifications Article number to which the sample refers.

The manufacturer's installation directions shall be provided with each sample. The Contractor shall pay all transportation costs and deliver samples to the Engineer's office, Site or testing laboratory as directed by the Engineer/Engineer’s Representative.

Samples shall be of adequate size to permit proper evaluation of the material by the Engineer. Where variations in colour, texture, dimensions or other characteristics are to be expected, the Contractor shall submit samples showing the maximum range of variation. Materials exceeding the range of variation of the approved samples shall not be used on the Work.

If both Shop Drawings and samples are required for the same item, the Engineer may require both to be submitted before approving either.

No acceptance or approval of any Shop Drawings or sample, or any indication or request by the Engineer on any Shop Drawings shall constitute an authorization for any increase in the Contract Sum unless specifically approved by the Engineer.

13. 	AS BUILT DRAWINGS

The Contractor shall, at all times, keep on site a separate set of prints on which shall be noted neatly, accurately and promptly as the work progresses all significant changes between the work shown on the drawings and that which is actually constructed.

At the completion of the works, the Contractor shall at his expense, supply the Engineer with three copies of these drawings.

14. 	PROTECTION OF THE WORKS

The Contractor shall whenever necessary cover up and protect the works from weather and damage by his own or other workmen performing subsequent operation. He shall provide all necessary dust sheets, barriers and guard rails and clear away the same at completion.

15. 	RESTORATION AND CLEANING

Upon completion of the works the Contractor shall restore all items covered by the Contract to the satisfaction of the Engineer.

The Contractor shall do regular cleaning and cleaning away all rubbish and excess materials that may accumulate from time to time on completion and before handing over. Upon completion of the works he shall obliterate all signs of temporary construction facilities such as work areas, structures, foundations of temporary structures, stock piles of excess or waste materials, or any other vestiges of construction, as directed by the Engineer.

16. 	CONSTRUCTION PROCEDURES

The Contractor shall advise the Engineer of proposed construction procedures in accordance with the General Conditions of Contract.

If the Engineer shall see that the work progress is slow in such away that the work will not be complete in the time specified, then he shall order the Contractor to work overtime or in more shifts and the Contractor shall obey these orders free from additional payments and without any objections or request for compensation.

17. 	NOTIFICATION TO ENGINEER

The Engineer shall be notified in writing of the nature and location of the Works the Contractor intends to perform the next 15 days so as to enable necessary inspection and measurement to be carried out. The Engineer may, if necessary, direct that longer notice be given of certain operations.

18. 	NIGHT WORK

When work is done at night the Contractor shall maintain from sunset to sunrise such lights on or about his work and plant as the Engineer may deem necessary for the proper observations of the work and the efficient execution thereof.

19. 	WEATHER

No work is to be undertaken when, in the opinion of the Engineer, the weather is so unsuitable that proper protection of the work cannot be ensured.

20. 	CO-ORDINATION WITH OTHER CONTRACTORS

It shall be the responsibility of the Contractor to co-ordinate and keep-up good relations with other Contractors employed on site by the Employer. The Contractor shall adjust his work plan to facilitate efficient performance of work being carried out by other contractors/persons at site.

21. 	ACCIDENT PREVENTION, PROTECTIVE EQUIPMENT

The Contractor shall comply and enforce compliance also by all his sub-contractors (if possible) with the highest standards of safety and accident prevention in accordance with international standards and in compliance with all applicable laws, ordinances and statutory provisions.

All requisite barriers, fences, warning signs, lights and other safety precautions as required for the protection of persons and property on or adjacent to the site shall be provided at the Contractor's cost.

All falsework, scaffolding and hand rails shall be well constructed and secured at all times. Where overhead work is being carried out, warning signs shall be installed at ground level clearly warning of the overhead work.

All warning signs shall be in two languages, English and Urdu, and shall at all times be maintained in a clean and legible condition, to the satisfaction of the Engineer/Engineer’s Representative.

Trash shall be removed at frequent intervals to the satisfaction of the Engineer/Engineer’s Representative.

1002 	TEMPORARY FACILITIES

The Contractor shall provide, erect or install, maintain alter as necessary and remove on completion when directed by the Engineer all temporary facilities and services including access roads as described hereinafter and/or in the Contract Document and/or instructed and approved by the Engineer.

01. 	Facilities for the Engineer

A. 	Site Office

As indicated in Sub-Clause 15.5, of Special Provisions of Contract.

B.	Transport

As indicated in Clause 16, of Special Provisions of Contract.

02. 	Temporary Roads

The Contractor shall construct, operate and maintain such temporary roads as may be necessary, from the site to the nearest road and also within the Project limits. Such roads shall be positioned strictly in accordance with the Engineer's instructions and the Contractor shall reduce or control any dust nuisance by spraying with water as directed.

03. 	Temporary Services

A. 	Temporary Water Supply

The Contractor shall supply in sufficient quantity all necessary potable and other water for construction purposes for all trades at point within a reasonable distance of any road or pavement being constructed. He shall make arrangements and pay charges for water service installation, maintenance and removal thereof, and pay the costs of water for all trades.

At completion of the work the temporary water services equipment and piping shall be removed by the Contractor at his own expense.

B. 	Temporary Electricity

The Contractor shall make all the necessary arrangements for a temporary electricity service, pay all expense in connection with the installation, operation and removal thereof and pay the costs of electricity consumed by all trades.

In the event that the site can not be connected to a local electricity network or where the available power is insufficient the Contractor has to make his own temporary provision for electricity generation and maintain such temporary installation.

A temporary lighting system shall be furnished, installed and maintained by the Contractor as required to satisfy the minimum requirements for safety and security and to the satisfaction of the Engineer/Engineer’s Representative.

Any expense, loss or damage suffered by the Contractor as a result of any interruption in power supply shall be born by the Contractor alone.

C. 	Waste Disposal

The Contractor shall make such temporary provisions as may be required in order to dispose of any chemicals, fuels, oils, grease, bituminous materials, waste and soil waste and the like without causing pollution to either the site or the environment. Disposal of any materials, wastes, effluents, garbage, oil, grease, chemicals and the like shall be in areas specified by the concerned local authority proposed by the Contractor and subject to the approval of the Engineer. If any waste material is dumped in unauthorized areas the Contractor shall remove the material and restore the area to the condition of the adjacent undisturbed area. If necessary, contaminated ground shall be excavated, disposed off as directed by the Engineer and replaced with suitable fill material compacted and finished with topsoil all at the expense of the Contractor.

Contractor shall provide concrete washout areas as specified in the specifications or as directed by the engineer.

D. 	Fire Protection

The Contractor shall provide and maintain adequate fire protection in the form of barrels of water with buckets, fire bucket tanks, fire extinguishers, or other effective means ready for instant use, distributed around the project and in and about temporary inflammable structures during construction of the works.

Gasoline and other flammable liquids shall be stored in and dispensed from safety containers approved by the Engineer and storage shall not be within building.

Torch-cutting and welding operations performed by the Contractor shall have the approval of the Engineer before such work is started and a chemical extinguisher is to be available at the location where such work is in progress.

The Contractor shall follow the instructions and specifications of the Civil Defence Department.

1003 	PAYMENT OF WORK

No payment shall be made for the works involved within the scope of this section of specification unless otherwise specifically stated in the Bills of Quantities or herein.

The cost thereof shall be deemed to have been included in the quoted unit rate of other items of the Bills of Quantities.